Drone detection methods in thermal imaging recordings using AI and classic computer vision methods

Table of Contents

- Introduction

- General Process of Drone Detection Using Thermal Imaging

- Detection Methods Overview
 3.1. Classical Computer Vision Techniques
 3.2. AI and Machine Learning Approaches

- Options for Drone Detection Using Thermal Imaging

- Summary and Recommendations

- References and Related Research

- Introduction

The purpose of this report is to explore the available approaches for detecting drones using thermal imaging and to evaluate which technologies are best suited for integration into a real-time localization system.

Thermal imaging technology allows us to spot objects by detecting the heat they give off instead of relying on visible light. This feature comes in handy, especially in situations where visibility is poor, like at night or during foggy weather. Lately, there's been a growing interest in using this technology to identify and track aerial objects, including drones. By utilizing thermal cameras, we can enhance security, surveillance, and air traffic monitoring. However, successfully detecting drones requires sophisticated image analysis techniques to differentiate them from other heat-emitting objects and background interference.

- General Process of Drone Detection Using Thermal Imaging

The process of detecting drones with thermal imaging can be broken down into several important steps:

- Thermal Image Acquisition – A thermal camera captures frames that reveal temperature differences in the environment, making drones visible as heat signatures against their surroundings.

- Preprocessing – The raw thermal data is cleaned up and adjusted to improve image quality and minimize background noise.

- Segmentation – The system identifies areas of interest (regions with notable temperature differences) that could indicate the presence of drones.

- Feature Extraction – Various attributes like shape, size, movement, and temperature patterns are examined to characterize the detected objects.

- Object Classification – The features that have been extracted are analyzed to figure out if an object is a drone or something else that emits heat.

- Localization and Tracking – After detection, the drone’s location and movement are monitored over time for ongoing observation or to work with other tracking systems.

- Detection Methods Overview

There are two main approaches to detecting drones in thermal imagery: traditional computer vision techniques and artificial intelligence (AI)-based methods. Both aim to identify and classify drones from thermal recordings, but they differ significantly.

3.1 Classical Computer Vision Methods

Short examples, advantages, limitations:

Thresholding + Contour Detection

Segment the image by setting a temperature (or brightness in thermal) threshold, then detect continuous regions (“blobs”) which are above that threshold. Analyze contours or shapes of those blobs to decide if they resemble a drone.

Motion Detection

Compare consecutive frames to find areas of change (moving heat sources). Movement helps to distinguish drones from static heat sources (e.g. metal, buildings).

Shape / Size Filtering

After detecting blobs, any that are irrelevant based on their size, shape, or density are discarded for example, those that are too small, too large, or too narrow.

Noise Reduction / Smoothing

Applying filters (Gaussian blur, median filter) to reduce sensor noise, followed by morphological operations if needed to clean up segmented regions.

Advantages:

  - Lightweight and fast execution.

  - Easier to interpret and debug.

  - Low requirements for training data.

Limitations:

  - Highly sensitive to environmental conditions.

  - Poor performance when drone is small, or in presence of cluttered background.

  - Manual tuning of thresholds / parameters required.

3.2 AI / Machine Learning Methods

Examples, model types, special challenges:

Object Detection Models (CNNs)

Models like YOLO (You Only Look Once), Faster R-CNN, SSD are often adapted for detecting drones. They are trained on sets of images where drones are labelled, and learn features of what a drone looks like in thermal imagery.

Transfer Learning

Using a model pre-trained on large datasets (often in visible light) and then fine-tuning it on thermal image datasets.

Advantages:

  - Higher detection accuracy in difficult conditions.

  - More robust to variability (shape, background, lighting).

  - Better detection of small drones if model architecture and training are well done.

Limitations:

  - Need for labelled thermal datasets.

  - Higher computational resources.

  - Possible latency or power consumption issues for real‐time use.

- Options for Drone Detection Using Thermal Imaging

When selecting a method for detecting drones with thermal cameras, several possible approaches exist. Each has trade-offs between accuracy, complexity, cost, and robustness. Below is a comparative table of potential strategies, followed by guidance on which solution might be appropriate given your priorities.

| Model | Framework / License | Performance (Speed / Accuracy) | Hardware Requirements | Strengths | Limitations |
| --- | --- | --- | --- | --- | --- |
| YOLOv8 (Ultralytics) | PyTorch / Open Source (AGPL-3.0) | Very high speed, high accuracy | Works on CPU or GPU (optimal on RTX GPU) | Modern architecture, excellent documentation, strong community support | Requires domain adaptation for thermal imagery |
| YOLOv5 (Ultralytics) | PyTorch / Open Source (GPL-3.0) | High speed, slightly lower accuracy vs YOLOv8 | CPU or GPU (supports older GPUs) | Mature ecosystem, stable, easy to train | Slower inference and older backbone |
| Faster R-CNN (TorchVision) | PyTorch / BSD License | Moderate speed, very high accuracy | High-end GPU recommended | Proven research-grade performance, excellent small-object detection | Computationally heavy, not ideal for real-time processing |
| SSD (Single Shot Detector) | PyTorch / MIT License | Fast, moderate accuracy | Runs well on mid-range GPU or CPU | Lightweight, easy to deploy on embedded systems | Poor detection of small or low-contrast objects |
| YOLO-NAS (Deci.ai) | PyTorch / Open Source | Excellent speed–accuracy tradeoff | GPU recommended (supports ONNX/TensorRT) | Optimized for edge devices, state-of-the-art performance | Requires Hugging Face account for download |
| Tiny YOLO / YOLOv8n | PyTorch / Open Source | Extremely fast, reduced accuracy | Works on CPU or small GPU | Ideal for rapid prototyping and embedded devices | Limited precision for complex scenes |

- Summary and Recommendations

In conclusion, both classical computer vision and AI-based methods offer valuable tools for detecting drones in thermal recordings. Traditional methods are fast and resource-efficient but struggle under complex environmental conditions. AI models, on the other hand, achieve higher detection accuracy and adaptability, particularly when fine-tuned for thermal imagery.

For our project, a practical starting point would be using a lightweight pre-trained AI model, combined with simple preprocessing to handle noise. As the project progresses, we can consider fine-tuning a model using available thermal drone datasets to improve detection precision and reliability.

- References and Related Research

- Ultralytics YOLO https://github.com/ultralytics/ultralytics

- Ultralytics YOLOv5 https://github.com/ultralytics/yolov5

- TorchVision Object Detection Models https://docs.pytorch.org/vision/stable/models.html#object-detection

- SSD.PyTorch https://github.com/amdegroot/ssd.pytorch

- Anti-UAV Dataset https://anti-uav.github.io/dataset

Additional Literature (For Further Reading)

- Thermal Imaging Drone Technology (2025). International Journal for Research Publication and Seminar. Overview of thermal imaging drones in various fields, including AI integration and neural network training for detection. [JRPS Journal]​

- Svanström, F., Englund, C., & Alonso-Fernandez, F. (2020). Real-Time Drone Detection and Tracking With Visible, Thermal and Acoustic Sensors. Conference paper on multisensor drone detection systems including thermal cameras. [ICPR Conference]​

- Barisic Kulas, A., Jurasovic, A., & Bogdan, S. (2013). Unlocking Thermal Aerial Imaging: Synthetic Enhancement of UAV Datasets. Methods for creating synthetic thermal datasets to improve object detection including drones in aerial thermal images. [arXiv]​

- Semenyuk, V., et al. (2025). Advances in UAV Detection: Integrating Multi-Sensor Data Augmentation for Thermal Imaging. Discusses data augmentation methods for thermal drone detection using neural networks. [ScienceDirect]​

- Lim, D. Y., et al. (2023). Heat-Vision Based Drone Surveillance Augmented by Deep Learning. Application of infrared imaging with deep learning for drone detection. [Nature]​

- Dahri, F. H., et al. (2024). Real-Time Detection of UAV Small Target's Infrared Signatures Using YOLOv8. Presents the OSTD-YOLOv8 model for small object detection in thermal UAV imagery with high precision. [IEEE]​