Analysis of the quality of database recordings and preprocessing testing

Introduction

The goal of this task was to analyze the quality of thermal camera frames and understand how basic preprocessing methods affect the visibility.
Two different datasets were used:

- thermal_samples1 (PNG images, 320×256)

- thermal_samples2 (JPG images, 640×512).

Each image was processed automatically using several filters.
The program calculates key image quality metrics — contrast, sharpness, and noise — both before and after preprocessing.
All numerical results are saved into a single CSV file.

Quality Metrics and Calculations

Contrast – calculated as the standard deviation of pixel intensity values (np.std(img)). Measures the spread of brightness values, higher contrast means more visible details.

Sharpness – measured using the variance of the Laplacian (cv2.Laplacian(img, cv2.CV_64F).var()). Estimates edge clarity, high values correspond to well-defined contours.

Noise – estimated by comparing the image with its Gaussian-blurred version. Mean absolute difference between the image and its blurred version.

For each processed image, the script saves:

- the original and processed values for each metric

- and the percentage change (Δ%)

Results Summary Table

| Filter Name | Δ Contrast % | Δ Sharpness % | Δ Noise % | General Effect |
| --- | --- | --- | --- | --- |
| equalizeHist | +105.3 | +452.9 | +224.1 | strong contrast and sharpness, very high noise |
| CLAHE | +7.6 | +93.4 | +98.8 | moderate enhancement, noise increase |
| median | −0.7 | −51.2 | −41.3 | noise reduction, blurred details |
| gaussian | −0.9 | −46.8 | −39.7 | smooths noise, loses detail |
| gamma | +1.5 | +28.4 | +17.6 | gentle contrast correction |
| unsharp | +0.4 | +70.7 | +25.9 | sharper look |
| bilateral | −1.6 | −78.9 | −67.3 | very strong denoising |
| non_local_means | −0.6 | −18.9 | −46.9 | best noise reduction |
| opening | −2.3 | −32.6 | −15.8 | removes small hot pixels |
| tophat | +3.8 | +64.1 | +33.5 | highlights local bright areas, in some cases could be useful |
| blackhat | +2.7 | +53.8 | +29.4 | Highlights dark areas, useless |

In general:

- For noise reduction, the most effective were non-local means and bilateral.

- For contrast and sharpness improvement, the most effective were equalizeHist, CLAHE, and unsharp masking.

- Combining one enhancement filter (CLAHE) with one denoising filter (non-local means) would improve the best overall image quality

Example
Photos in order: original, equalizeHist, CLAHE, median, gaussian, gamma, unsharp, bilateral, non_local_means, opening, tophat, blackhat