YOLOv5s – Tests on Thermal Imaging Data

# Dataset

- 300 images captured with a thermal camera

  - 290 images at 640×512 resolution

  - 10 images at 320×256 resolution

Command used:

python detect.py --weights yolov5s.pt --source thermal_images_chinese/ --project results --name thermal_images_chinese --exist-ok

- 5 videos, each with 300+ frames, resolution 512×640

Command used:

python detect.py --weights yolov5s.pt --source thermal_images/ --project results --name thermal_images --exist-ok

# Model

- YOLOv5s pre-trained on COCO dataset

- Detection threshold: conf = 0.25

# Results

Videos

IR_AIRPLANE_001

- Airplane detected in one frame (confidence ~0.36)

- Remaining detections incorrect

IR_BIRD_013

- No detections

IR_DRONE_026

- No detections

IR_DRONE_082

- No detections

IR_HELICOPTER_011

- No detections, except a background building incorrectly classified as a train

Images

- For images with 320×256 resolution → no detections

- Overall detection statistics:

  - 27 airplanes (most of which were actually drones)

  - 11 trains (false positives – background buildings)

- Typical confidence range: 0.28–0.40

# Example images results:

# Conclusions

- YOLOv5s, pre-trained on COCO, does not include a “drone” class.

  - Some drones were still detected but mislabeled as airplanes.

- The COCO dataset contains color (RGB) images, while our data is infrared (thermal), which strongly reduces model performance.

- IR images in this task are not well suited for an out-of-the-box COCO-trained model.

- YOLOv5s is optimized for a 640×640 input size, which is the default and recommended resolution in the official Ultralytics configuration. Using lower or non-square resolutions (such as 512×640 or 320×256) further decreases detection accuracy, especially for small objects.

# Next steps:

Fine-tune or retrain the model using a labeled ground-truth thermal dataset, and then re-evaluate performance.