Drone Localization and Future Position Prediction

Technical Development Report

1. Project Goal

The goal of this project was to design and implement a drone localization and prediction system based on:

- thermal video input,

- object detections produced by a YOLOv5 model,

- classical tracking using an Extended Kalman Filter (EKF),

- prediction of the drone’s future position approximately 1 second ahead,

- visualization of:

  - past trajectory,

  - predicted future position,

  - prediction uncertainty.

The system was required to work without CUDA/ROCm GPU acceleration, due to hardware limitations (AMD GPU without ROCm support), which enforced a CPU-only inference and post-processing pipeline.

2. Initial Setup and Constraints

2.1 Hardware and Software Constraints

- AMD GPU without ROCm support

- No CUDA support available

- YOLOv5 inference performed offline

- Detection results saved as YOLO-format .txt label files

- Tracking and prediction performed using Python + NumPy + OpenCV

2.2 Input Data

Two different input formats were used during development:

- Video-based input

  - .mp4 thermal video

  - Resolution: initially assumed 320×256, later corrected to 640×512

  - FPS: varied (24–60 FPS)

- Frame-based input

  - Folders containing:

    - images: 000001.jpg, 000002.jpg, ...

    - labels: 000001.txt, 000002.txt, ...

  - YOLO label format:

  - class_id x_center_norm y_center_norm width_norm height_norm

3. Initial Tracking Implementation

3.1 EKF-Based Drone Tracker

The tracker was implemented as a custom DroneTracker class, internally using an Extended Kalman Filter with the following state vector:

[x, y, v, yaw, yaw_rate]

Where:

- (x, y) – drone position in image coordinates

- v – velocity magnitude

- yaw – motion direction

- yaw_rate – angular velocity

The EKF provided:

- state prediction per frame,

- state correction using detections,

- multi-step forward prediction for future localization.

4. First Version – Correct Trajectory, Incorrect Prediction

4.1 Observed Behavior

- The past trajectory (green line) was drawn correctly.

- The predicted position (blue point + ellipse) was always drawn exactly on the drone.

- Prediction did not appear to move 1 second into the future.

4.2 Root Cause

The prediction logic was technically correct, but:

- the EKF state contained zero or near-zero velocity,

- velocity and direction were never explicitly estimated,

- future prediction collapsed to the current position.

5. Second Version – Improved Prediction, Broken Trajectory

5.1 Attempted Fix

Velocity and yaw were estimated manually using the last two history points:

dx = history[-1][0] - history[-2][0]

dy = history[-1][1] - history[-2][1]

v = sqrt(dx^2 + dy^2) / dt

yaw = atan2(dy, dx)

These values were injected directly into the EKF state.

5.2 Result

- Prediction became coherent and moved forward in time.

- However:

  - the trajectory became unstable,

  - history points were no longer consistent,

  - state updates and visualization became desynchronized.

5.3 Explanation

The issue was caused by:

- mixing measured positions with predicted EKF states,

- overwriting EKF state variables manually every frame,

- computing velocity from only two frames, which amplified noise.

6. Final Solution – Multi-Frame Motion Estimation

6.1 Key Insight

Reliable motion estimation requires:

- averaging over multiple past frames,

- separating:

  - measurement update,

  - state prediction,

  - visualization history.

6.2 Improved Approach

- Velocity and direction are estimated using a sliding window of N past frames (e.g. 10).

- Motion is computed as:

  - average displacement over multiple frames,

  - divided by elapsed time.

- EKF state is updated consistently.

- Trajectory history stores true filtered positions, not predictions.

This resulted in:

- stable trajectory visualization,

- realistic future prediction,

- smooth uncertainty ellipse.

7. Visualization Improvements

7.1 Transparency and Readability

To avoid occluding the drone in thermal imagery:

- semi-transparent drawing was introduced using alpha blending,

- thinner lines and shapes were used.

This applied to:

- trajectory line,

- predicted point,

- uncertainty ellipse.

8. Dataset Conversion Pipeline

8.1 Problem

Some datasets were provided without video files, only as:

- image sequences (.jpg)

- detection labels (.txt)

8.2 Solution

A separate script was implemented to:

- Convert image sequences into a single .mp4 video

- Copy and normalize YOLO label files into a ready-to-use labels directory

- Allow configurable:

  - FPS

  - resolution

This enabled reuse of the same tracking pipeline across all datasets.

9. Critical Debugging Issues and Fixes

9.1 Relative Path Errors

- Python was executed from a different working directory than expected.

- Relative paths to videos and labels failed silently.

Fix:
Use absolute paths or derive paths from __file__.

9.2 Resolution Mismatch

- YOLO labels are normalized relative to original image resolution.

- Using incorrect width/height caused all detections to be misplaced or ignored.

Fix:
Always compute pixel coordinates using actual video frame dimensions.

9.3 Tracker Never Initializing

The message:

detection is None and not tracker.initialized

indicated that:

- no detection was ever successfully loaded,

- EKF was never initialized,

- tracking never started.

Fix:
Correct label paths, naming convention, resolution usage, and frame indexing.

10. Final Outcome

After all fixes and refinements:

- The tracker initializes correctly from the first detection.

- Past trajectory is accurate and stable.

- Prediction is correctly projected ~1 second into the future.

- Uncertainty visualization reflects motion dynamics.

- The pipeline works for:

  - .mp4 videos

  - image + label folders

- The solution is fully CPU-compatible and hardware-agnostic.

11. Project Status

The system now represents a complete classical tracking + prediction pipeline, suitable for:

- academic projects,

- capstone demonstrations,

- hybrid ML + control systems research.

It demonstrates:

- understanding of Kalman filtering,

- practical debugging of real-world data pipelines,

- correct handling of coordinate systems, timing, and uncertainty.